# Concrete Engineering: Grade-Wise Admixture Cost Analysis (₹/m³)

This report converts technical dosages into practical financial impacts per cubic meter of concrete, specifically for the Indian RCC market.

## 1. Primary Assumptions for Calculation

Calculations are based on standard Indian Site/RMC practices (IS 456 & IS 10262).

| Grade | Estimated Cement (kg/m³) | W/C Ratio (Target) | Use Case |
| --- | --- | --- | --- |
| M20 | 350 kg | 0.50 – 0.55 | Foundations, small slabs |
| M25 | 380 kg | 0.45 – 0.50 | Standard columns/beams |
| M30 | 420 kg | 0.40 – 0.45 | High-rise structural RCC |
| M40 | 480 kg | 0.35 – 0.40 | Heavy Infra, Bridges, HPC |

## 2. Chemical Admixture Cost Breakdown

Based on typical market rates: Superplasticizers (₹150–250/kg) and Plasticizers (₹80–150/kg).

### A. Superplasticizer (PCE Based - Mandatory for Pumpable RCC)

Dosage assumed at 1.0% of cement weight.

- M20: 3.5 kg × ₹150–250 = ₹525 – ₹875 / m³

- M25: 3.8 kg × ₹150–250 = ₹570 – ₹950 / m³

- M30: 4.2 kg × ₹150–250 = ₹630 – ₹1,050 / m³

- M40: 4.8 kg × ₹150–250 = ₹720 – ₹1,200 / m³

### B. Specialized Additives

- Accelerators/Retarders: (1.5% dosage) ≈ ₹400 – ₹4,000 / m³ (Extreme weather/Mass concrete).

- Air Entraining Agents: (0.03% dosage) ≈ ₹50 – ₹150 / m³ (Pavements/Dams).

## 3. Mineral Admixture Cost Breakdown

Mineral admixtures significantly alter the "green" cost of concrete.

### A. Fly Ash (Class F)

Replacement: 25% of cement.

- Cost: ₹200 – ₹475 / m³ (Logistics based).

- Impact: Reduces overall m³ cost by replacing expensive OPC 53 cement.

### B. GGBS (Slag)

Replacement: 40% of cement.

- Cost: ₹500 – ₹1,200 / m³

- Impact: Drastically improves sulfate/chloride resistance (Marine structures).

### C. Silica Fume (Micro-Silica)

Replacement: 8% of cement.

- Cost: ₹950 – ₹2,300 / m³

- Impact: Essential for M60+ or low-permeability M40.

## 4. Final Combined Additive Cost (Summary)

Total estimated cost added to the base materials (Sand/Aggr/Cement).

| Concrete Grade | Typical Additive Mix | Total Additive Cost (₹/m³) |
| --- | --- | --- |
| M20 (Normal) | Plasticizer + Fly Ash | ₹500 – ₹1,200 |
| M25 (Standard) | Superplasticizer + Fly Ash | ₹700 – ₹1,400 |
| M30 (Pumpable) | High-Range SP + GGBS | ₹1,000 – ₹2,000 |
| M40 (High Strength) | Premium SP + Silica Fume | ₹1,800 – ₹3,200 |
| FRC (Special) | SP + Steel Fibers (25kg) | ₹3,000 – ₹4,500 |

## 5. Engineering Insights

- Cost Efficiency: While Silica Fume or Graphene increases m³ cost, they allow for "Slender Design" (less concrete volume), potentially reducing total project cost.

- Workability: For Pumpable concrete (100mm+ slump), the Superplasticizer cost is non-negotiable and typically accounts for 8–12% of the total concrete price.

- Brand Variation: Premium brands like Fosroc (Auramix) or Sika (Viscocrete) typically command a 15-20% premium over local chemical blenders but offer better consistency in strength gain.