# Engineering Comparison: Traditional RCC vs. Graphene-Enhanced Concrete

This report provides a practical, engineer-level comparison between traditional Reinforced Cement Concrete (RCC) and Graphene-based systems (e.g., Graphacrete, Monoatom).

## 1. Baseline RCC Cost (Including Additives)

Estimated Ready-Mix/Site-Mix costs in India for the 2025–2026 period.

| Grade | Base RCC Cost (₹/m³) | Additive Mix Type | Additive Cost (₹/m³) | Total RCC Cost (₹/m³) |
| --- | --- | --- | --- | --- |
| M20 | ₹4,500 – ₹5,200 | Plasticizer + Fly Ash | ₹150 – ₹300 | ₹4,700 – ₹5,500 |
| M25 | ₹5,000 – ₹5,800 | Superplasticizer + Fly Ash | ₹250 – ₹450 | ₹5,300 – ₹6,200 |
| M30 | ₹5,800 – ₹6,800 | High-range SP + GGBS | ₹400 – ₹800 | ₹6,200 – ₹7,600 |
| M35 | ₹6,800 – ₹7,800 | SP + Silica Fume | ₹700 – ₹1,200 | ₹7,500 – ₹9,000 |
| M40 | ₹7,800 – ₹9,000 | Premium SP + SF + Fibers | ₹1,000 – ₹2,000 | ₹8,800 – ₹11,000 |

## 2. Graphene / Graphacrete Market Reality

Graphene admixtures are nano-engineered solutions that significantly alter the hydration matrix of cement.

### Key Performance Indicators

- Dosage: 0.05% – 0.5% of cement weight.

- Strength Gain: Up to 50% increase in compressive strength.

- Cement Reduction: Potential to reduce cement content by 20% – 30%.

- Curing: Up to 25% faster curing times.

### Practical Market Estimates (India Bulk Usage)

| Graphene Admixture Type | Cost Impact (₹/m³) |
| --- | --- |
| Low-cost graphene admixture | ₹300 – ₹800 |
| Mid-range graphene mix | ₹800 – ₹1,500 |
| High-end nano-graphene | ₹1,500 – ₹3,000 |

## 3. Financial Case Studies: Normal vs. Graphene

### Case 1: Same Strength (Cost Optimization)

Goal: Use Graphene to reduce cement while maintaining Grade.

- M30 (Normal): ₹6,500/m³

- M30 (Graphene): ₹6,200 – ₹6,800/m³

- Result: 5% – 20% potential cost saving through cement reduction.

### Case 2: Higher Strength (Performance Optimization)

Goal: Maintain cement levels to reach higher performance.

- Normal M30: ₹6,500/m³

- Graphene M30 (Behaves like M40): ₹7,200 – ₹8,500/m³

- Result: Higher initial cost but achieves significantly higher strength and durability.

### Case 3: Premium High-Performance Concrete (HPC)

- Conventional M50: ₹10,000 – ₹12,000/m³

- Graphene HPC: ₹9,500 – ₹11,500/m³

- Result: Often cost-neutral or cheaper per unit of strength.

## 4. Performance vs. Cost Summary

| Parameter | Normal RCC | Graphene Concrete (Graphacrete) |
| --- | --- | --- |
| Strength | Standard Baseline | +20% to +50% |
| Cement Usage | 100% | 70% – 80% |
| Crack Resistance | Medium | Very High |
| Durability | 30 – 50 Years | 50 – 100 Years |
| Initial Cost/m³ | Base | ±0% to +15% |
| Life-cycle Cost | High | Low |

## 5. Engineering Conclusion

Goal: Low-Cost Construction

- Solution: Standard Fly Ash + GGBS + Superplasticizer.

- Cost: ₹200 – ₹800 / m³ (additive impact).

Goal: Premium / Infrastructure / Future-Proofing

- Solution: Graphene / Graphacrete.

- Strategic ROI: Reduced structural weight (smaller columns), less steel reinforcement, and a significantly lower carbon footprint.

### Reality Check

The Graphene market is emerging; prices vary based on project scale. ROI is highest when structural redesign is performed to account for the increased strength (e.g., using less concrete and steel overall).